Tamiya PS paints are specialized spray paints designed for flexible polycarbonate (Lexan) RC car bodies. They remain flexible after curing, resist cracking when the body flexes, and are fuel- and oil-resistant, making them ideal for both electric and gas-powered RC vehicles.

## What Tamiya PS Paints Are

- PS = Polycarbonate Spray: Specifically formulated for transparent polycarbonate shells used in radio-controlled (RC) cars.

- Flexible Formula: Unlike standard paints, PS paints maintain elasticity after drying, preventing cracks or peeling when the RC body bends during use.

- Fuel & Oil Resistance: Safe for nitro/gas-powered RC cars, as the paint withstands exposure to fuel and lubricants.

- Application Method: Always sprayed on the inside of the clear polycarbonate shell. This protects the paint from scratches and gives a glossy, durable finish.

## Key Features

- Can Size: Each spray can contains 100ml, enough for multiple RC body coatings.

- Drying Time: Touch-dry in 15–20 minutes, fully cured in about 24 hours.

- Color Range: Over 40+ PS colors available, including metallics, fluorescents, and standard shades (e.g., PS-2 Red, PS-5 Black, PS-12 Silver).

- Finish Quality: Produces smooth, even coverage with high pigment density for vivid, authentic colors.

## How They Are Used

- Preparation: Wash the polycarbonate shell to remove oils and dust.

- Spraying: Hold the can about 30 cm away and apply light, even coats inside the shell.

- Layering: Apply multiple thin coats for depth and durability. Metallic or translucent colors often require backing with black or silver for proper effect.

- Detailing: Small details can be brush-painted using Tamiya’s bottle paints for polycarbonate.

- Protection: Because the paint is applied inside, the exterior remains clear and resistant to scratches.